RETINAL VESSEL SEGMENTATION USING IMPROVED MATCHED FILTERING : EVALUATION ON A HIGH RESOLUTION FUNDUS IMAGE DATABASE.

INTRODUCTION:

Automatic  assessment  of  retinal  vessels  plays  an  important  role  in  the  diagnosis  of various  eye,  as  well  as  systemic  diseases. A  public  screening  is  highly  desirable  for prompt  and  effective  treatment,  since  such  diseases  need  to  be  diagnosed  at  an  early stage.  Automated  and  accurate  segmentation  of  the  retinal  blood  vessel  tree  is  one   of  the  challenging  tasks  in  the  computer-aided  analysis  of  fundus  images  today.  We improve  the  concept  of  matched  filtering,  and  propose  a  novel  and  accurate  method   for segmenting  retinal  vessels.  Our  goal  is  to  be  able  to  segment  blood  vessels  with  varying vessel  diameters  in  high-resolution  colour  fundus  images.  All  recent  authors  compare  their  vessel  segmentation  results  to  each  other  using  only  low-resolution  retinal  image databases.  Consequently,  we  provide  a  new  publicly  available  high-resolution  fundus image  database  of  healthy  and  pathological  retinas.  Our   performance  evaluation  shows that  the  proposed  blood  vessel  segmentation  approach  is  at  least  comparable  with  recent state-of-the-art  methods.  It  outperforms  most  of  them  with  an  accuracy  of  95%  evaluated on  the  new  database.

OBJECTIVES OF THE PROJECT:

Digital imaging using a fundus camera is widely considered an integral part of medical examination in ophthalmology. Standard fundus images contain various regions which can

be useful for diagnosis, such as the macula, which is usually examined in connection with age-related macular degeneration; the optic disc for examination of glaucoma; and vascular structures, which are mostly evaluated in the context of diseases affecting the circulatory

system

Several pathologies affecting the retinal vascular structures due to diabetic retinopathy can be found in fundus images using precisely segmented blood vessels. Moreover, automatic retinal vessel segmentation algorithms can be useful in the evaluation of other diseases, such as arteriolar narrowing and vessel tortuosity due to hypertensive retinopathy or glaucoma.

METHODS

Previously, low resolution fundus images were used for the research analysis. The proposed method uses a publically available high resolution dataset of fundus images for analysis and produces more accurate results. This  database  contains   three  sets  of  fundus  images : one  of

Healthy  retinas , one  of  glaucomatous  and  one  of  DR  retinas. The  second  and  third  groups ,  thus ,  allow   evaluation   of  the  segmentation   methods   in   the  case  of   pathological  retinas.

The  preprocessing   step  consists  of  illumination  correction  and  Contrast  equalisation of the fundus  images  in  preparation  for  further  analysis.  Only  the  green  channel  of  an  RGB image  is  utilised,  since  this  channel  has  the highest  contrast  between blood vessels and other  retinal  structures. The   segmentation  of  blood  vessels  in  the  preprocessed  image  utilises  a  MF  approach.  Five  two-dimensional   filters  were  designed  according  to  typical blood  vessel  cross-sectional  intensity  profiles, whereas  five  different  blood  vessel  widths

were   considered – from  the  thinnest  to  the  thickest  retinal  vessels.  The   preprocessed image  is  convolved  with  each  of   the   five   kernels,  each  of which  is  rotated  into  12 different  orientations.  The  resulting  parametric  images  are  then  fused  so  that  the  locally maximum   response  is  selected  for  each  pixel. The  fused  parametric  image  is  then thresholded   to   obtain   a binary map of the blood vessel tree. This is thenfurther cleaned using morphological  operators  to  remove  small  or  short  artefacts  due  to  noise  or  other  image

structures  that  do  not  belong  to  the  vascular  tree.

EXPECTED RESULTS

The  presented  blood  vessel  segmentation  method  was  primarily  evaluated  using  the  new    HRF  database  containing Hand - labelled  images.